# M-Duino-PCSM

Project brief for the Parameter Control and Status Monitoring application.

| Document date | 2026-05-02 |
| --- | --- |
| Project title | M-Duino-PCSM |
| Subtitle | Parameter Control and Status Monitoring |
| Purpose | Capture the technical, architectural, and styling decisions required before implementation. |

# 1. Purpose

Create a desktop or web-style control interface for the M-Duino test controller that allows the operator to change approved parameters between runs, monitor controller status, and view state transitions while preserving the institute visual identity used in the DIME Toolbox.

# 2. Agreed System Architecture

The M-Duino remains the real controller. The Python application is only a supervisory layer and must not directly run the real-time firing sequence.

M-Duino responsibilities:

arming logic

firing sequence logic

hot-wire timing

spark timing

DAQ timing

fail, abort, and fired lockout behavior

parameter validation and safety limits

PC application responsibilities:

parameter editing

parameter readback

live status display

event and transition log display

serial communication with the M-Duino

# 3. Hardware and Firmware Baseline

Target hardware: Industrial Shields M-Duino 19R+

Original reference code: M-Duino/ArduinoCode/M-Duino_Original/M-Duino_Original.ino

Current cleaned controller code baseline: M-Duino/ArduinoCode/M_Duino_v002/M_Duino_v002.ino

The cleaned controller code should be treated as the firmware baseline for app integration because it already includes explicit timing variable names with units, commented sequence logic, named controller states, clearer output/flag separation, and debug-oriented state transition logging.

# 4. Connection and Communication Assumptions

The app connects to the M-Duino controller itself, not directly to the hot-wire power supply, spark hardware, or DAQ hardware.

Primary connection method: USB serial

The M-Duino remains powered from its proper external supply.

The USB link is used for parameter exchange, status monitoring, and event logging.

# 5. Critical Interface Requirement

The app and firmware can only work together if they share a stable external interface. The app does not edit compiled binary values directly. Instead, the firmware must expose a serial command vocabulary that maps to internal variables.

Firmware variable example: hotWireBurn_us

Protocol name example: HOTWIRE_US

Operator-facing label example: Hot-wire burn time

# 6. Initial Parameter Mapping

| Firmware variable | Protocol name | Operator label | UI unit | Notes |
| --- | --- | --- | --- | --- |
| hotWireBurn_us | HOTWIRE_US | Hot-wire burn time | s | Main tuning parameter |
| sparkDwell_us | SPARK_US | Spark dwell | us | Spark pulse duration |
| daqPulse_us | DAQ_US | DAQ pulse width | us | Must not exceed spark dwell |
| sparkTestInterval_us | SPARKTEST_US | Spark-test interval | ms | Used in spark-test mode |
| useHotWireStep | USE_HOTWIRE | Use hot-wire step | bool | Enables or skips melting stage |

# 7. Initial Status Contract

At minimum, the firmware should expose enough information for the app to display:

current controller state

current operating mode

current Arm input state

current Trigger input state

last transition reason

whether the loaded parameter set was accepted

Recommended additional status fields:

Spark output active

DAQ output active

hot-wire step enabled

startup configuration summary

firmware version identifier

# 8. Serial Protocol Direction

The planned communication approach is a simple text-based serial protocol.

Example commands:

GET ALL

STATUS?

SET HOTWIRE_US 15000000

SET SPARK_US 5000

SET DAQ_US 600

SET SPARKTEST_US 500000

SET USE_HOTWIRE 1

Example responses:

OK

ERROR NOT_IDLE

ERROR OUT_OF_RANGE

VALUE HOTWIRE_US 15000000

STATE IDLE

MODE HYDROGEN_TEST

ARM 0

TRIGGER 0

REASON operator reset after fired state

Parameter updates should only be accepted when the controller is in a safe state such as IDLE.

# 9. App and Firmware Behavior Requirements

The application should:

detect available serial ports

let the operator choose the correct M-Duino port

connect and disconnect cleanly

request current values on connect

show the latest controller state continuously

show whether a parameter update was accepted or rejected

show the returned reason or error message

keep a readable event log

avoid allowing unsafe parameter edits in the UI

The firmware should:

expose the parameter interface over serial

expose live status over serial

expose readable reasons for transitions or rejections

enforce safe-state restrictions for parameter changes

validate all incoming values

keep the firing logic local to the M-Duino

provide a stable naming convention for all exposed values

# 10. Planned Python App Stack

Python

Panel

pyserial

This stack was chosen because it is open source, flexible, and well suited for a parameter-and-status dashboard.

# 11. Visual Design Direction

The app should follow the institute style already used in the DIME Toolbox.

Institute logo included in the header

Inter as the main UI font

JetBrains Mono for technical or console-style text where useful

warm off-white page background

warm light gray surfaces/cards

charcoal text

charcoal top bar

restrained crimson accent

minimal shadows

crisp borders

professional research-tool feel

# 12. Design Tokens Extracted from DIME Toolbox

| Element | Value |
| --- | --- |
| Page background | #F7F7F5 |
| Surface/card | #EEEEEA |
| Main text | #333333 |
| Muted text | #888888 |
| Top bar | #2E2E2E |
| Secondary chrome | #3A3A3A |
| Accent crimson | #CC2222 |
| Accent light | #F9E5E5 |
| Border tone | #D1D1CD |

# 13. Proposed App Sections

Connection panel

Editable parameter panel

Live status panel

Event / transition log

# 14. First-Version Success Criteria

Connect to the M-Duino over USB serial

Read the current parameter set

Display the current controller state and core input status

Send a new parameter value while the controller is in IDLE

Receive and display acceptance or rejection from the firmware

Show a readable event or transition log

Preserve the institute visual identity from DIME Toolbox

# 15. Next Recommended Steps

Create the GitHub repository for M-Duino-PCSM

Move or copy this brief into that repository

Write the formal serial interface specification

Update the M-Duino firmware to support safe runtime parameter updates

Create the first Python Panel UI scaffold

Add the institute logo and DIME-style theme tokens to the app